📦 COSHH PACK – REFRIGERANTS USED IN FACTORY REFRIGERATION INSTALLATION

Company: PR Cooling Ltd
Project: TBC
Date: TBC

1. COSHH SHEET – R404A (A1 – NON-FLAMMABLE)

Substance Name: R404A

Type: HFC Blend

Classification: A1 – Non-flammable, high GWP

Hazards

- Asphyxiation in confined spaces

- Frostbite from liquid contact

- High pressure – risk of cylinder rupture

- Environmental impact (very high GWP)

Exposure Routes

- Inhalation

- Skin/eye contact

Control Measures

- Use in well-ventilated areas

- Leak detection equipment

- Proper recovery equipment

- Avoid confined spaces

PPE

- Gloves

- Safety glasses

- Long sleeves

Emergency Actions

- Evacuate area

- Ventilate

- Seek medical attention for frostbite or inhalation

2. COSHH SHEET – R452A (A1 – NON-FLAMMABLE)

Substance Name: R452A

Type: HFO/HFC Blend

Classification: A1 – Non-flammable

Hazards

- Asphyxiation risk

- Frostbite from liquid contact

- High pressure system

Exposure Routes

- Inhalation

- Skin/eye contact

Control Measures

- Use calibrated gauges

- Ensure ventilation

- Leak testing required

- Avoid confined spaces

PPE

- Gloves

- Safety glasses

- Protective clothing

Emergency Actions

- Evacuate

- Ventilate

- Treat frostbite with lukewarm water

- Seek medical attention

3. COSHH SHEET – R134a (A1 – NON-FLAMMABLE)

Substance Name: R134a

Type: HFC

Classification: A1 – Non-flammable

Hazards

- Asphyxiation in high concentrations

- Frostbite

- Decomposition at high temperatures can produce toxic gases

Exposure Routes

- Inhalation

- Skin/eye contact

Control Measures

- Use in ventilated areas

- Leak detection

- Avoid heating refrigerant lines

PPE

- Gloves

- Safety glasses

Emergency Actions

- Evacuate

- Ventilate

- Seek medical attention

4. COSHH SHEET – CO₂ (R744) – HIGH PRESSURE REFRIGERANT

Substance Name: CO₂ (R744)

Type: Natural refrigerant

Classification: A1 – Non-flammable, very high pressure

Hazards

- Extreme pressure – risk of explosion if mishandled

- Asphyxiation in confined spaces

- Cold burns/frostbite

- Rapid expansion can displace oxygen

Exposure Routes

- Inhalation

- Skin/eye contact

Control Measures

- Use pressure-rated equipment

- Never heat cylinders

- Ensure ventilation

- Monitor oxygen levels in enclosed areas

PPE

- Insulated gloves

- Safety glasses

- Face shield (recommended)

Emergency Actions

- Evacuate area

- Ventilate

- Treat frostbite

- Seek medical attention

5. COSHH SHEET – R290 (PROPANE) – A3 HIGHLY FLAMMABLE

Substance Name: R290 (Propane)

Type: Hydrocarbon

Classification: A3 – Highly flammable

Hazards

- Explosion risk

- Highly flammable vapour

- Frostbite from liquid contact

- Asphyxiation in confined spaces

Exposure Routes

- Inhalation

- Skin/eye contact

Control Measures

- No hot work in area

- Intrinsically safe tools only

- Ventilation mandatory

- Leak detection (hydrocarbon-rated)

- Eliminate ignition sources

PPE

- Anti-static clothing

- Gloves

- Safety glasses

Emergency Actions

- Evacuate immediately

- Do NOT operate electrical switches

- Ventilate area

- Fire service if ignition occurs

- Treat frostbite and seek medical attention

This COSHH Pack will be reviewed annually or at any time, where a significant change requires a review.

Signed by: Rob Chappell		Signature:

Health & Safety Consultant  	Dated: 28/03/2026			Next Review: 12 Months