Location Address:

Number of Installers:

Method Statement Date:Click or tap to enter a date.

Method Statement: Installation of Air Conditioning Systems in Office Premises

1. Scope of Works

This method statement covers the safe installation of split-system or multi-split air conditioning units within an office environment. Works include:

- Positioning and mounting indoor fan coil units

- Installing outdoor condenser units

- Routing and securing refrigerant pipework and condensate drains

- Electrical connections and commissioning

- Testing and handover

2. Responsibilities

Site/Project Manager

- Ensure all operatives are competent and briefed

- Confirm risk assessment and method statement are understood

- Coordinate with building management and office occupants

Installation Engineers

- Carry out installation in accordance with this method statement

- Use tools and equipment safely

- Report hazards or issues immediately

Electrician (Qualified)

- Complete all electrical isolations and final connections

- Test and certify electrical works

3. Resources Required

Labour

- 1 × Air conditioning engineers (Minimum)

- 1 × Electrician (as required)

Plant & Equipment

- Ladders or mobile tower scaffold

- Vacuum pump and gauges

- Pipe bender and flaring tools

- Drill and fixings

- Leak detection equipment

- Lifting aids (as required)

Materials

- Indoor and outdoor AC units

- Refrigerant pipework and insulation

- Condensate drain

- Electrical cabling and isolators

- Brackets and fixings

4. Sequence of Works

4.1 Pre-Start

- Attend site induction

- Review RAMS with team

- Confirm work area is clear and safe

- Isolate electrical supplies where required

- Protect office furniture and equipment

4.2 Installation of Indoor Unit

- Mark out mounting position ensuring clearances are met

- Check wall for hidden services using a detector

- Fix mounting plate securely

- Drill pipe and cable routes to outside or riser

- Mount indoor unit and connect pipework, drain, and control cable

4.3 Installation of Outdoor Unit

- Position unit on brackets or slab ensuring stability

- Maintain required airflow clearances

- Secure unit and apply anti-vibration mounts if needed

- Route and insulate pipework between indoor and outdoor units

- Connect electrical supply and interconnecting cable (electrician)

4.4 Refrigerant & Commissioning

- Pressure-test pipework

- Evacuate system using vacuum pump

- Release or charge refrigerant as required (F-Gas engineer only)

- Check for leaks

- Power up system and test operation

- Set controls and confirm performance

4.5 Completion

- Remove waste and reinstate work area

- Re-enable any isolated fire alarms

- Demonstrate system operation to client

- Provide commissioning sheet and certification

5. Emergency Procedures

- Follow site emergency arrangements

- Report incidents immediately to the site manager

- In case of refrigerant leak: ventilate area and evacuate if necessary

- For electrical incidents: isolate supply and seek first aid

6. Environmental Considerations

- Prevent refrigerant release to atmosphere

- Dispose of waste materials responsibly

- Minimise noise and disruption to office occupants

7. PPE Requirements

- Safety boots

- Gloves (general and refrigerant-safe)

- Safety glasses

- Hearing protection (as required)

- Hi-vis clothing (if working near vehicle routes)

This method statement will be reviewed annually or at any time, where a significant change requires a review.

Signed by: Rob Chappell		Signature:

Health & Safety Consultant		Dated: 28/03/2026	Next Review: 12 Months