TOOLBOX TALK

| TOOLBOX TALK TITLE | Portable, Hand-held electrical tools |
| --- | --- |
| REASON FOR TALK | Electrical tools face harsh conditions and when misused they get damaged and become dangerous. |
| WHY? | In one year, there were 194 reported incidents of electric shock involving portable electrical tools. |
| OUTLINE | This toolbox talk covers pre-use checks, use of electrical tools & hazards. |

Pre-use checks

- Make sure the casing isn’t damaged and, if it is, do not use it

- Make sure all cables, plugs or connectors are not damaged and if you have to replace a fuse, ensure you use the correct / same amperage.

- Use tools on the correct power supply. Generally, only 110 volt tools are permitted on sites.

- Check the tool has a current PAT test sticker on it and it is PAT tested annually.

Safe use

- Portable electrical tools should only be used for their designated purpose.

- Ensure switches are working correctly before connecting to the power supply or charging.

- Wear eye protection if there is any risk to your eyes, for example drilling.

- Disconnect tools when not in use, or remove the battery, even if making adjustments.

- Electrical power tools should be regularly inspected and maintained by a competent electrican.

Hazards

- If possible, keep power cables off the floor. They may get damaged and cause someone to trip, or they may trail through water.

- Electrical tools often present a noise hazard. Wear hearing protection if necessary.

- Keep all power tools and equipment clean and dry.

- Portable electrical tools that have become wet should be be allowed to dry and then be checked by a competent person for electrical safety before being used again.

- Some hand-held tools can cause hand-arm vibration.

- When charging power tools batteries ensure that you use the original charger only and it is in a safe environment when in use, IE: Clean, dry and well ventilated.

Always follow the manufacturers instructions on the safe use and maintence of all power tools.

Never use a power tool if you can see any damage to the tool, cables or chargers, this could cause a high risk of electric shock.