Section 1. The Work Details:

Address where the work will be taking place: Various Locations

Is this a Construction Site:  YES:		NO:

If YES, Enter Employee(s) CSCS Card Numbers Here:

| Employee(s) Name: | CSCS Card Number: | Expiry Date: |
| --- | --- | --- |

Section 2. The Area of Risk & Exposure:

| Area / Item Assessed: | Installation of air conditioning units | Manual handling Involved: | YES |
| --- | --- | --- | --- |
| Location: | Various Locations | COSHH Requirement: | Yes |
| The Process: | Installation and commissioning of air conditioning units | Nosie Assessment: | No |
| Equipment To Be Used: | Ladders – see working at height. Various electronic and manual tools – see manufacturers instructions. | Policy Requirement: | Manual Handling Working at Height COSHH Hot Works |
| Substances Involved: | Nitrogen Refrigerants | Exposure Time: | Up to 15 minutes |
| Exposed Persons: | Employees | Other: | N/A |
| Third Party Fitters |
| Other Operatives |

Section 3. Identified Hazards:

| HAZARD IDENTIFIED: | BEFORE CONTROLS: |  |  |  | AFTER CONTROLS: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | L | M | H |  | L | M | H |
| Manual handling |  |  | X |  |  | X |  |
| Unloading equipment |  |  | X |  | X |  |  |
| Slips, trips or falls |  |  | X |  | X |  |  |
| Risk of electric shock |  |  | X |  | X |  |  |
| Working at height |  | X |  |  | X |  |  |
| Injury from power or manual tools |  |  | X |  |  | X |  |
| Exposure to substances |  | X |  |  | X |  |  |
| Exposure to Asbestos |  |  | X |  |  | X |  |
| Hot working – use of naked flames |  |  | X |  |  | X |  |
| Location must be Covid secure |  | X |  |  | X |  |  |

Section 4. Control Measures:

| REF NO: | CONTROL MEASURE / INSTRUCTION: |
| --- | --- |
| 1 | Ensure your vehicle is in a safe condition and your vehicle safety check has been completed prior to attending site, drive within the conditions and regulations. |
| 2 | Park in a safe manner or designated location when attending sites, where possible this should be a hard standing and level. |
| 3 | When carrying equipment ensure T.I.L.E.S is followed, only carry what you can safely, making additional trips if required. Ensure all equipment is kept in a safe location, ensuring there is no trip hazard and the location is dry. |
| 4 | Any chemicals or gases must be stored upright, kept away from any naked flame or heat source and stored securely in-line with the manufacturers instructions. Employees must have up to date data sheets with all chemicals to be used and trained in their safe use. |
| 5 | Power tools are only to be used by employees who are 'compentant' in their use, this will be judged by training and experience in the use of the tools. |
| 6 | Any accidents or product damages must be reported to the employees line manager and H&S Consultant without delay. |
| 7 | If Hot Works are to be carried out a hot works certificate must be in place for the correct times as required and all instructions including fire breaks must be followed. |
| 8 | Ladder safety checks must be completed prior to using any ladders and must only be used by a trained and ‘competent’ employee. |
| 9 | Cherry pickers or scaffolding must only be used by suitably trained and competent employees. Areas must be sealed off from members of the public and third parties and all safety gates must be in the down or locked positions when in use. |
| 10 | All safety devices, dead switches and gates must be checked prior to use to ensure they are fully working. Safety harnesses must be used where instructed. Working at height policy must be followed and in use. |
| 11 | Areas were working at height is in operation must be kept clear of other workers, members of the public and where possible signage in place to show working at height is in operation. If the area is in the open it must be barriered off with no lone working. |
| 12 | Any access to areas containing asbestos must be cleared by a professional first and asbestos registers must be confirmed for the site prior to works starting. |
| 13 | Lifting new equipment must be done carefully, ensuring employees only lift what they can safely manage and inline with manufacturers instructions. |
| 14 | Electrical systems must be isolated and locked off or labelled not to be used. Installers are to check power supplies are sufficient and safe with no visible damage for before connecting any new units. Any damage must stop work and be reported. |
| 15 | All manufacturers instructions must be followed for the safe installation of units, connection of pipe works. All must be double checked prior to commissioning. |
| 16 | Social distancing, face masks must be in use, regular hand washing and sanitiser must be in place. Employees must comply with site location Covid-19 rules. |

Section 5. Training of Employees:

All employees that are listed above will have completed the following training within the last 12 months.

Manual Handling Training

Asbestos Awareness Training

Working from Heights

Vehicle Safety

Gas Safe, if required for hot works

Site Safety – For those who carry CSCS Cards.

Section 6. The Method Statement’s identified for use with this Risk Assessment are:

| REF NO: | TITLE: |
| --- | --- |
| 1 | Manual handling |
| 2 | Working at height safely |
| 3 | COSHH |
| 4 | Hot works policy & certification |
| 5 | Covid-19 company information |

Monitoring Results:

All risk assessments will be reviewed on an annual basis, if an incident is reported during this time period the review will take place as part of any investigation, updates made will be uploaded to the one drive system with a reference to all staff involved.

LOLER Examinations will be completed on an annual basis, unless otherwise required by Law. Certificates of all examinations are held at our head office.

Departmental Managers are responsible for full compliance of this risk assessment at all times.

This Risk Assessment has been authorised for use by:

Rob Chappell – Health & Safety Consultant.

Dated: 18/04/2022

Next Review is due by: 18/04/2023