Location Address:

Number of Installers:

Risk Assessment Date:Click or tap to enter a date.

Risk Assessment – Repairs to Chiller Units

1. Activity

Repairs, servicing, and component replacement on commercial/industrial chiller units, including electrical diagnostics, refrigerant handling, mechanical repairs, and functional testing.

2. Persons at Risk

- PR Cooling Ltd engineers

- Other contractors on site

- Site staff and visitors

- Members of the public (if in accessible areas)

3. Hazard Identification & Control Measures

⚡ Electrical Hazards

Risks: Electric shock, arc flash, burns, unexpected energisation.
Controls:

- Full isolation using site LOTO procedures.

- Verify dead before starting work.

- Only competent, qualified engineers to access electrical panels.

- Use insulated tools and appropriate PPE.

- Replace panel covers before re-energising.

❄ Refrigerant Exposure (R32, R410A, etc.)

Risks: Frostbite, asphyxiation in confined spaces, flammability (R32), environmental release.
Controls:

- F-Gas certified engineers only.

- Leak testing before opening system.

- Use recovery equipment for refrigerant removal.

- Adequate ventilation around plant.

- No hot works near flammable refrigerants.

- Correct PPE: gloves, goggles, long sleeves.

- Follow COSHH guidance for specific refrigerants.

🔧 Mechanical Hazards

Risks: Contact with rotating fans, pumps, belts, sharp edges, pressurised components.
Controls:

- Isolate mechanical systems before working.

- Allow system pressures to equalise before opening circuits.

- Guarding replaced immediately after work.

- Use correct tools and lifting aids for heavy components.

🔥 Hot Works (if required)

Risks: Burns, fire ignition, damage to insulation or nearby materials.
Controls:

- Hot works permit where applicable.

- Fire watch and extinguishers on hand.

- Remove combustibles from work area.

- No hot works near refrigerant lines containing flammable gases.

🪜 Working at Height

Risks: Falls from ladders, platforms, or roof-mounted chillers.
Controls:

- Use fixed access or certified mobile platforms.

- Maintain three points of contact on ladders.

- Edge protection on roofs where possible.

- Avoid working at height during adverse weather.

🏗 Manual Handling

Risks: Strains, sprains, back injuries from lifting compressors, coils, panels.
Controls:

- Assess weight before lifting.

- Use mechanical lifting aids (hoists, trolleys).

- Team lifts for awkward items.

- Keep work area clear to avoid slips/trips.

🌡 High/Low Temperature Surfaces

Risks: Burns from hot discharge lines; frostbite from cold suction lines.
Controls:

- Allow system to stabilise before contact.

- Wear thermal gloves.

- Insulate exposed pipework after repairs.

🧯 Fire & Explosion

Risks: Ignition of refrigerant, electrical fire, overheating components.
Controls:

- No smoking near plant.

- Ensure correct fire extinguishers available (CO₂/dry powder).

- Check for refrigerant leaks before energising.

- Maintain clear access to emergency exits.

🧹 Slips, Trips & Falls

Risks: Tools, hoses, panels, or refrigerant oil on the floor.
Controls:

- Keep work area tidy.

- Use drip trays for oil.

- Good housekeeping throughout task.

- Adequate lighting.

🔊 Noise Exposure

Risks: Hearing damage from running compressors or plantroom acoustics.
Controls:

- Hearing protection where noise levels exceed safe limits.

- Limit time spent near running plant.

🚚 Site-Specific Hazards

Examples:

- Traffic movement

- Restricted access

- Confined plantrooms

- Roof edge hazards
Controls:

- Site induction

- Follow site rules

- Dynamic risk assessment on arrival

4. Residual Risk Rating

With controls in place, residual risk is Low to Medium, depending on site conditions and task complexity.

5. Required PPE

- Safety boots

- Gloves (mechanical and thermal)

- Safety glasses/goggles

- Hearing protection (as required)

- Hi-vis clothing

- Respiratory protection if required by COSHH assessment

6. Competence Requirements

- F-Gas certification

- Electrical competence (18th Edition or equivalent)

- Manufacturer-specific training where applicable

- Awareness of site safety rules and RAMS

This risk assessment will be reviewed annually or at any time, where a significant change requires a review.

Signed by: Rob Chappell		Signature:

Health & Safety Consultant   Dated: 28/03/2026			Next Review: 12 Months